PAW3950 driver wrapper for ZMK

This is an ZMK pointer input module, that grant ability to call a non-disclosed PAW3950 driver library.

This driver had been tested on my PAW3395 breakout board.

Note

PAW3950 pinout is kind of compatible to PAW3395.

Caution

This module is build for nRF52840 Cortex-m4 MCU. Only tested on Xiao BLE. Any register address and control value regarding to PAW3950 datasheet is not available in this module under PixArt's NDA.

Installation

Include this project on ZMK's west manifest in config/west.yml:

manifest:

remotes:

...

# START #####

- name: badjeff

url-base: https://github.com/badjeff

# END #######

...

projects:

...

# START #####

- name: zmk-paw3950-driver

remote: badjeff

revision: main

# END #######

...

self:

path: config

Update board.overlay adding the necessary bits (update the pins for your board accordingly):

&pinctrl {

spi2_default: spi2_default {

group1 {

psels = <NRF_PSEL(SPIM_SCK, 1, 13)>,

<NRF_PSEL(SPIM_MOSI, 1, 15)>,

<NRF_PSEL(SPIM_MISO, 1, 14)>;

};

};

spi2_sleep: spi2_sleep {

group1 {

psels = <NRF_PSEL(SPIM_SCK, 1, 13)>,

<NRF_PSEL(SPIM_MOSI, 1, 15)>,

<NRF_PSEL(SPIM_MISO, 1, 14)>;

low-power-enable;

};

};

};

#include <zephyr/dt-bindings/input/input-event-codes.h>

&spi2 {

compatible = "nordic,nrf-spim";

status = "okay";

pinctrl-0 = <&spi2_default>;

pinctrl-1 = <&spi2_sleep>;

pinctrl-names = "default", "sleep";

cs-gpios = <&gpio1 12 GPIO_ACTIVE_LOW>;

pd0: pd@0 {

compatible = "pixart,paw3950";

status = "okay";

label = "MOU0";

reg = <0>;

spi-max-frequency = <4000000>;

// paw3950 driver parameters

irq-gpios = <&gpio1 11 (GPIO_ACTIVE_LOW | GPIO_PULL_UP)>;

cpi = <600>;

// swap-xy; /* optional */

// invert-x; /* optional */

// invert-y; /* optional */

evt-type = <INPUT_EV_REL>;

x-input-code = <INPUT_REL_X>;

y-input-code = <INPUT_REL_Y>;

};

};

/ {

mouse_input_listener {

compatible = "zmk,input-listener";

device = <&pd0>;

};

};

Enable the driver config in <shield>.config file (read the Kconfig file to find out all possible options):

CONFIG_SPI=yCONFIG_INPUT=yCONFIG_ZMK_POINTING=yCONFIG_PAW3950=y# CONFIG_PAW3950_SWAP_XY=y // <-- deprecated, use swap-xy; instead# CONFIG_PAW3950_INVERT_X=y // <-- deprecated, use swap-xy; instead# CONFIG_PAW3950_INVERT_Y=y // <-- deprecated, use swap-xy; instead# CONFIG_PAW3950_REPORT_INTERVAL_MIN=12# CONFIG_PAW3950_LOG_LEVEL_DBG=y# CONFIG_PAW3950_INIT_POWER_UP_EXTRA_DELAY_MS=3000

About

No description, website, or topics provided.

Resources

Readme

Activity

Stars

1 star

Watchers

0 watching

Forks

0 forks

Report repository

Releases

No releases published

Packages

No packages published

Contributors1

- badjeff

Languages

- C97.7%

- CMake2.3%

Footer

© 2026 GitHub, Inc.